ASIMOV v1

ARCHITECTURE DESCRIPTION DOCUMENT

Concept-phase · System boundary · Functional decomposition · Safety architecture · CAN topology · Mode FSM

Document control

| Field | Value |
| --- | --- |
| Document ID | ASV1-ARCH-CP-001 |
| Revision | 0.1 (draft) |
| Issue date | 2026-05-03 |
| Author | J. Herrod-Thomas |
| Item | Asimov v1 humanoid robot — github.com/jherrodthomas/asimov-v1 |
| Companion artefacts | ASV1-SC-CP-001 Safety Case · ASV1-DFMEA-CP-001 DFMEA · ASV1-REQ-CP-001 Requirements |
| Standards basis | ISO/IEC/IEEE 42010:2022 · ISO 12100 §5 · ISO 13482 §5 · ISO 13849-1 §6 |
| Confidentiality | Public — distributed under CERN-OHL-S-2.0 reciprocity |

Important — concept-phase architecture This document describes the architecture as inferred from the upstream repository (CAD subassemblies, KiCad sources, WireViz harness, MuJoCo model, manual). Where the upstream documentation is silent — for example, the wireless E-Stop internal architecture — assumptions are flagged and traced to the Gap Register in the Safety Case workbook.

1. Introduction

This document describes the concept-phase architecture of the Asimov v1 open-source humanoid robot. It supplements the Preliminary Safety Case (ASV1-SC-CP-001), the DFMEA (ASV1-DFMEA-CP-001), and the Requirements Specification (ASV1-REQ-CP-001). The architecture is presented through five complementary views:

| View | Diagram | Purpose |
| --- | --- | --- |
| Context | Diagram 1 — System boundary | Item, actors, external interfaces, hazard zones (ISO 12100 §5) |
| Functional | Diagram 2 — Functional block | Subsystem decomposition with power, CAN, signal flows |
| Safety | Diagram 3 — SF allocation map | 25 safety functions allocated to HW / SW / procedure with PL targets |
| Communication | Diagram 4 — CAN bus topology | Six CAN buses with node lists and termination strategy |
| Behavioural | Diagram 5 — Mode FSM | DAMP / STAND / MOVE state machine with watchdog and E-Stop gating |

1.1 Architecture style and decomposition principles

Asimov v1 is decomposed by physical subassembly (mechanical) and by logical board (electrical), with the Motion Control Board (MCB) acting as the central kHz-rate controller and the Raspberry Pi 5 (RPi5) acting as a media and tele-op gateway. The split mirrors the safety architecture: hard real-time and safety-critical functions live on the MCB; rich-OS, network and media functions live on the RPi5. Communication between MCB and the limbs uses six CAN buses partitioned by limb to limit the blast-radius of a single bus failure (one bus-off → one limb lost, not the whole robot).

1.2 Architectural assumptions

| ID | Assumption | Trace |
| --- | --- | --- |
| AA-01 | Each limb is on its own CAN bus; bus-off on one bus does not affect the other limbs | SYRS-022 · SF-08 |
| AA-02 | MCB and RPi5 are on separate power rails with separate brown-out detection | HWRS-016 |
| AA-03 | Wireless E-Stop receiver is independent of MCB — it cannot be vetoed by FW | HWRS-019 · GAP-13482-04 |
| AA-04 | Battery (when fitted) authenticates to the MCB before motor rails enable | SYRS-014 · SRS-018 |
| AA-05 | Locomotion policy is software-only and runs as a clamped output through SF-03 | SRS-006 · GAP-LP-01 |
| AA-06 | The kit ships pre-crimped harness — DIY users do not crimp safety-critical connections | FM-18 · HWRS-012 |
| AA-07 | Operator tele-op session is authenticated end-to-end via TLS 1.3 | SYRS-010 · SRS-007 |

2. System boundary and context

The system boundary follows ISO 12100 §5 (determination of the limits of the machinery) and ISO 13482 §5 (personal-care robot scoping). External actors include the trained operator, bystanders (only relevant outside OE-1), the operating environment, the maintainer, the power source (battery or bench supply), the cloud / network providing WebRTC tele-op, the operator app, the charger, supply-chain partners, and the regulatory authority for any deployment outside an internal lab.

Figure 1 — System boundary and context per ISO 12100 §5 / ISO 13482 §5.

2.1 External interfaces

| Interface | Direction | Medium | Safety relevance |
| --- | --- | --- | --- |
| Operator → tele-op | in | Wi-Fi / 4-5G WebRTC | SAFE — auth + idle lock (SYRS-010, SRS-007) |
| Operator → E-Stop | in | Wireless RF | SAFE Cat 1 — link surveillance (SYRS-003, HWRS-019) |
| Robot → operator app | out | WebRTC media + data | Telemetry + camera + audio |
| Bystander exposure | — | Physical (reach) | Hazard zones (HZ-M01..M07, HZ-P02..P06) |
| Environment | in | Air, surface | +10..+35 °C, ≤5° slope, dry indoor |
| Maintainer assembly | — | Procedural | Lift / PPE / ESD (SF-12, SF-17, SF-18) |
| Power source | in | DC via XT30 | SF-10 isolation, SF-15 BMS handshake |
| Charger | in | Mains AC | External — assumed compliant per AA-04 |
| OEM / supplier | — | Procurement | Material certs (SYRS-028) |
| Regulator | — | Compliance | CE / FCC / national robotics frameworks (out of scope this rev) |

2.2 Operating envelope

The architecture supports four declared operating envelopes; only OE-1 (lab supervised, tethered) is provisionally permitted at this revision:

| OE | Description | Risk | Acceptance |
| --- | --- | --- | --- |
| OE-1 | Lab, supervised, tethered, no bystanders | LOW | PROVISIONAL YES |
| OE-2 | Industrial cobot, defined cell | MEDIUM | Not yet — 12 gaps |
| OE-3 | Home / consumer, untrained users, children/pets | HIGH | Not yet — 19 gaps |
| OE-4 | Public space, free roaming | VERY HIGH | Not yet — 21 gaps + regulatory engagement |

3. Functional architecture

The functional view shows the seven mechanical subassemblies (ASV1_100 through ASV1_700) plus the four core electronics modules (RPi5 edge, MCB, PDB, Wireless E-Stop receiver) plus user-supplied battery and BMS, and the sensors block (camera, IMU, mics, encoders, thermistors). Connections are categorised as power (red), CAN (teal), or signal (grey).

Figure 2 — Functional block diagram showing mechanical subassemblies, electronics, and the six CAN buses.

3.1 Subsystem responsibilities

| Subassembly / module | Function | Allocated reqs |
| --- | --- | --- |
| ASV1_100 Pelvis / hip yoke | Carry full body weight + walking moments; house BMS bus stub | HWRS-002, FM-01 |
| ASV1_200 Left leg | 6 DOF + toe; left-side bipedal locomotion | F-200, SF-01 |
| ASV1_300 Right leg | 6 DOF + toe; right-side bipedal locomotion (mirror of 200) | F-300 |
| ASV1_400 Left arm | 5 DOF; payload to 18 kg lateral | F-400, SYRS-020 |
| ASV1_500 Right arm | 5 DOF; mirror of 400; uses SLM 316L parts | F-500, FM-30 |
| ASV1_600 Torso / waist | 1 DOF waist yaw; hosts MCB + 10 W speaker | F-600, SYRS-015 |
| ASV1_700 Head / neck | 2 DOF neck; hosts camera, mic-array, RPi5 | F-700, FM-29 |
| MCB (Radxa CM5) | Run motion-control kHz loop, 6 CAN, watchdog, host IMU | F-MCB, SF-08, SF-23, SF-24 |
| RPi5 | Media, network, WebRTC tele-op gateway, authentication | F-RPI, SF-21, SF-22 |
| PDB | 5 V + motor rail distribution, per-rail fuse, UVLO + PG | F-PDB, SF-10, SF-11 |
| Wireless E-Stop receiver | Latching motor-power contactor; link surveillance | SF-13, HWRS-019 |
| Battery + BMS (user) | Energy storage with cell-voltage / temp / current cutoff | SF-15, GAP-BMS-01 |
| Sensors | Image, audio, IMU, joint encoders, thermistors | F-SEN |

4. Safety architecture

Twenty-five safety functions are declared in the Functional Safety Concept (Safety Case tab 09_FSC). They are allocated to hardware (9), software / firmware (13), or procedure (3). The diagram below makes the allocation explicit and tags each function with its required Performance Level per ISO 13849-1.

Figure 3 — Safety function allocation map: HW (left) / SW (centre) / Procedural (right) with PL targets.

4.1 PL distribution

The aggregate Performance Level distribution across the 25 SFs is:

| PL / Cat | Count | Implication |
| --- | --- | --- |
| PL e / Cat 4 | 1 | BMS — single-fault tolerance with self-test |
| PL d / Cat 3 | 9 | Stability, controlled descent, torque/velocity limit, PFL, watchdog stop, E-Stop link, IMU plausibility, scheduler watchdog, BMS interlock |
| PL c / Cat 2 | 5 | Speed reduction, overcurrent, stator over-temp, joint software limits, auth |
| PL b / Cat 1 | 2 | Audio limiter, idle lock |
| Static / no PL | 8 | Inherent design or procedure (chamfering, guarding, PPE, lift) |

4.2 Cat 3 architecture rationale

Per ISO 13849-1 §6.2.6, a Cat 3 architecture requires that a single fault does not lead to loss of the safety function, and that single faults are detected whenever reasonably practicable. For Asimov v1 this implies:

| SF | Required Cat 3 implementation | Status / gap |
| --- | --- | --- |
| SF-01 Stability | Locomotion policy + IMU plausibility + actuator torque-monitor cross-channel | GAP-13849-01 — single-channel today |
| SF-02 Controlled descent | Torque-monitor + IMU plausibility + redundant stop path | GAP-13849-01 |
| SF-03 Torque / velocity limit | Drive-side limit + FW limit + cross-monitor | Partial — drive limit present, cross-monitor missing |
| SF-08 Watchdog safe-stop | HW watchdog + FW watchdog + independent contactor open path | Partial — HW watchdog present, contactor TBD |
| SF-13 Wireless E-Stop | Two independent receivers OR dual decoding path → latching contactor | GAP-13482-04 + GAP-10218-03 — architecture undocumented |
| SF-15 BMS | BMS with two independent protection MOSFETs + temperature / voltage cross-check | Deferred to user-supplied pack — GAP-BMS-01 |
| SF-23 IMU plausibility | Two independent IMUs OR IMU + commutation cross-check | GAP-13849-01 — single IMU on MCB today |
| SF-24 Scheduler watchdog | FW deadline monitor + HW WDT + independent stop path | Partial — both watchdogs present, stop path TBD |

5. Communication architecture — six CAN buses

Asimov v1 partitions actuator and BMS communication across six CAN buses. The choice limits blast-radius (a single bus-off cannot disable both legs) and matches the Asimov manual electrical specification. The MCB hosts three native CAN controllers and three SPI-CAN bridges.

Figure 4 — CAN bus topology, six buses with 120 Ω termination at both ends.

5.1 Bus mapping and rates

| Bus | Rate | Nodes (typical) | Owning subassembly |
| --- | --- | --- | --- |
| C0 — Left leg | 1 Mbps | Hip yaw / roll / pitch · Knee · Ankle pitch / roll · Toe sense | ASV1_200 |
| C1 — Right leg | 1 Mbps | Hip yaw / roll / pitch · Knee · Ankle pitch / roll · Toe sense | ASV1_300 |
| C2 — Right arm | 1 Mbps | Shoulder pitch / roll / yaw · Elbow · Wrist yaw | ASV1_500 |
| C3 — Left arm | 1 Mbps | Shoulder pitch / roll / yaw · Elbow · Wrist yaw | ASV1_400 |
| C4 — Waist-neck | 1 Mbps | Waist → Neck-yaw → Neck-pitch (daisy chain) | ASV1_600 / 700 |
| C5 — BMS | 500 kbps | BMS controller with cell-protection telemetry | SS-BMS |

5.2 Failure-mode strategy

On bus-off, the FW shall (a) attempt single recovery and (b) on second occurrence within 1 s transition to DAMP (SRS-011). Loss of the BMS bus C5 disables motor rails (SYRS-014). Bus-off → DAMP latency must be ≤ 100 ms (SF-08).

6. Behavioural architecture — Mode FSM

The robot operates in three modes — DAMP, STAND, MOVE — with deterministic transitions gated by the watchdog and wireless E-Stop. The FSM enforces that motion only occurs in MOVE, that MOVE entry requires authentication and a valid heartbeat, and that any safety-critical fault transitions immediately to DAMP.

Figure 5 — Mode FSM with transitions, watchdog gates, and fatal-fault paths.

6.1 Transition guard table

| From | To | Trigger | Guard conditions |
| --- | --- | --- | --- |
| (power-on) | DAMP | Boot complete | No torque applied; self-tests run (SRS-023) |
| DAMP | STAND | ModeCommand(STAND) | IMU plausibility OK · E-Stop armed · auth session active · 2 s ramp |
| STAND | MOVE | ModeCommand(MOVE) | Heartbeat < 250 ms · joint limits · torque limit · auth session |
| MOVE | STAND | ModeCommand(STAND) | Operator-initiated; controller transitions to standing pose |
| STAND | DAMP | Watchdog timeout / cmd | Any of: heartbeat lost, fault, manual command |
| MOVE | DAMP | FATAL FAULT | Any of: E-Stop pressed · torque-limit breach · IMU implausible · loop-deadline miss · CAN bus-off · BMS protection |

6.2 Reaction-time budgets

| Scenario | Detection | Reaction | Total budget |
| --- | --- | --- | --- |
| Heartbeat lost | ≤ 250 ms | ≤ 100 ms (SF-08) | ≤ 350 ms |
| Wireless E-Stop press | ≤ 50 ms | ≤ 500 ms (SF-13) | ≤ 550 ms |
| Wireless E-Stop link loss | ≤ 1000 ms | ≤ 1000 ms | ≤ 2 s — to fail-safe |
| Loop deadline miss | ≤ 50 ms | ≤ 50 ms (SF-24) | ≤ 100 ms |
| CAN bus-off | Immediate (interrupt) | ≤ 100 ms (SRS-011) | ≤ 100 ms |
| IMU implausibility | ≤ 50 ms (SF-23) | ≤ 50 ms | ≤ 100 ms |
| Stator over-temp | Continuous | ≤ 200 ms (SF-14) | ≤ 200 ms |

7. Architecture-to-safety-case trace

Each architectural element is traced back to the safety case for verifiability:

| Architectural element | Safety-case reference | DFMEA failure modes | Open gaps |
| --- | --- | --- | --- |
| MCB watchdog + scheduler | SF-08, SF-24 | FM-10, FM-13 | GAP-13849-01, GAP-13849-03 |
| Wireless E-Stop receiver | SF-13 | FM-19, FM-20 | GAP-13482-04, GAP-10218-03 |
| Battery + BMS | SF-15 | FM-21, FM-22 | GAP-BMS-01 |
| Locomotion policy + IMU | SF-01, SF-02, SF-23 | FM-04, FM-12, FM-28 | GAP-LP-01, GAP-13849-01 |
| Joint torque/velocity | SF-03, SF-04, SF-20 | FM-05, FM-06, FM-08 | GAP-13482-02, GAP-15066-01 |
| Six CAN buses + harness | SF-08 | FM-11, FM-18 | GAP-13849-01 |
| Power distribution | SF-10, SF-11 | FM-16, FM-17, FM-31 | GAP-60204-01 |
| Tele-op auth + cyber | SF-21, SF-22 | FM-15, FM-32 | GAP-CYB-01 |
| Mechanical structure | SF-05, SF-07 | FM-01, FM-02, FM-03, FM-30 | GAP-9001-03, GAP-13482-03 |

7.1 Conclusion

The architecture is internally consistent and the five views together provide a complete description of the item for concept-phase review. Significant safety-architecture gaps remain — most importantly the Cat 3 redundancy of SF-01/02/08/13/15/23/24 and the published architecture of the wireless E-Stop. These are tracked in the Safety Case Gap Register (tab 12_Gaps).

7.2 References

Standards: ISO/IEC/IEEE 42010:2022 · ISO 12100 §5 · ISO 13482 §5/§6 · ISO 13849-1 §6 · ISO 10218-1 §5. Project: github.com/jherrodthomas/asimov-v1; manual.asimov.inc/v1. Companion artefacts: Asimov_v1_Safety_Case.xlsx, Asimov_v1_DFMEA.xlsx, Asimov_v1_Requirements.xlsx.