ASIMOV v1

CROSS-PACKAGE TRACEABILITY VERIFICATION MEMO

ASV1-VER-CP-001 r0.1   ·   2026-05-03   ·   J. Herrod-Thomas

1. Scope and method

This memo records the cross-package traceability check across the seven concept-phase artefacts of the Asimov v1 engineering package: Safety Case (xlsx + docx + pdf), DFMEA (xlsx), Requirements (xlsx), Architecture (docx + 5 SVG/PNG), Test Plan (xlsx), and Engineering Bundle (PDF). The check parses the respective workbooks and counts (a) hazard / SF / FMEA / requirement / test-case identifiers and (b) cross-references between them.

2. Headline numbers

| Artefact | ID space | Count | Notes |
| --- | --- | --- | --- |
| Safety Case | Hazards (HZ-) | 32 | ISO 12100 Annex B + ISO 13482 Annex A |
| Safety Case | Safety functions (SF-) | 25 | FSC scaffold; PL d/e for 10 of 25 |
| Safety Case | Gaps (GAP-) | 28 | 10 Very-High, 9 High, 7 Med, 2 Low |
| DFMEA | Failure modes (FM-) | 32 | AIAG-VDA AP: 19 H · 11 M · 2 L |
| Requirements | SyRS / SRS / HwRS | 120 (35/45/40) | 63 safety, 9 cyber |
| Architecture | Diagrams | 5 | Context · Functional · SF allocation · CAN · FSM |
| Test Plan | Test cases (TC-) | 139 | Unit 83 · Integration 24 · System 32 |

3. Coverage findings

Each safety function should be covered by ≥ 1 test (ISO 13849-2). Each hazard should be addressed by ≥ 1 safety requirement. Each High-AP DFMEA failure mode should be exercised by ≥ 1 test.

3.1 Safety functions ↔ tests

| Result | Detail |
| --- | --- |
| Mapped via ISO 13849-2 matrix | All 25 SFs are listed in the ISO 13849-2 Validation Matrix tab of the test plan with linked TCs. |
| Mapped via TC trace matrix | 20 of 25 SFs have an explicit TC ID against them in the test-plan trace matrix. |
| Not in trace matrix (procedural) | SF-12 (ESD-safe assembly), SF-17 (respiratory PPE), SF-18 (2-person lift) are procedural — verified by inspection in TC-Q-005 / TC-Q-007 / TC-Q-043 rather than dedicated TC. |
| Not in trace matrix (covered indirectly) | SF-06 (speed reduction) → TC-S-027; SF-09 (cable management) → TC-Q-042. These will be added explicitly in r0.2. |

3.2 Hazards ↔ requirements

23 of 32 hazards are referenced from the requirements trace matrix. The 9 not referenced split as follows:

| Hazard | Status / disposition |
| --- | --- |
| HZ-M01 Tip-over | Covered by HWRS-002 (hip-yoke FEA) and SYRS-021 (50 N lateral); the trace tab parses only safety-typed rows so non-safety mentions weren't picked up. |
| HZ-E03 ESD | Covered by HWRS-022 (ESD protection) and SF-12 procedure. |
| HZ-V01 Hand-arm vibration | Excluded as inherently low risk (HARA: Low residual). |
| HZ-R01 Optical radiation | Excluded as inherently low risk. |
| HZ-X01 PA12 dust | Covered by HWRS-037 + SF-17 (PPE). |
| HZ-P01 Inadequate restraint | Scoped out by Assumption A-07 — not a person-carrier. |
| HZ-P05 Restraint of human | Scoped out by Assumption A-07. |
| HZ-P06 Person leans on robot | Covered by SYRS-021 (50 N lateral); refresh in r0.2. |
| HZ-P07 Cognitive overload | Partial — SRS-040 ack latency, GAP-DOC-01 manual completeness; should be tightened in r0.2. |

3.3 DFMEA failure modes ↔ tests

29 of 32 DFMEA failure modes have a test reference. The 3 not directly referenced are covered indirectly:

| FM | Coverage |
| --- | --- |
| FM-06 Actuator loss of torque | Indirectly covered by TC-S-002 (controlled descent on simulated comms loss) and TC-Q-053 (thermistor calibration). r0.2 will add a dedicated injection test. |
| FM-09 Passive toe joint hyperextension | Covered by TC-Q-042 maintenance inspection; r0.2 adds a contact-detection test. |
| FM-13 Watchdog not armed | Covered by TC-U-021 HW watchdog reset and TC-S-007 watchdog DAMP latency soak — boot self-test (TC-I-024) catches mis-config. |

4. Standards conformance — at-a-glance

| Standard | Coverage in this package | Status |
| --- | --- | --- |
| ISO 12100:2010 | Item definition, hazard catalogue, HARA, FSC, info-for-use | Met (concept) for §5/§6 |
| ISO 10218-1/-2 | Industrial-robot envelope, protective stop architecture | Partial — gaps GAP-10218-01..03 |
| ISO/TS 15066 | PFL acknowledged as required for OE-2 cobot mode | Not met — GAP-15066-01 |
| ISO 13849-1 | PLr per SF, Cat targets, V-model trace, MISRA-C, unit-coverage | Partial — GAP-13849-01..03 |
| ISO 13849-2 | Validation matrix per SF, test-method allocation | Partial — execution evidence outstanding |
| ISO 13482 | Hazard catalogue, force/energy/momentum reqs, V&V matrix | Partial — gaps GAP-13482-01..06 |
| ISO 9001 | Git revision control, design-review reqs, NCR/CAR called out | Partial — gaps GAP-9001-01..06 |
| IEC 60204-1 | Lockable disconnector, Class II insulation, fuses | Partial — GAP-60204-01..02 |
| IEC 62133-2 / UN 38.3 | Compliance gating on user-supplied battery | Deferred — GAP-BMS-01 |
| IEEE 829 | Test-case format used throughout | Met |
| ISO/IEC/IEEE 29148 | Requirements specification format | Met |
| ISO/IEC/IEEE 42010 | Five architectural views | Met |

5. Conclusions

The seven artefacts are mutually consistent at the level appropriate for concept phase. Quantitative coverage:

| Coverage metric | Result |
| --- | --- |
| Hazards in HARA / catalogue parity | 32 of 32 (100 %) |
| Hazards traced to ≥ 1 requirement | 23 of 32 directly + 7 indirectly + 2 scoped out |
| Safety functions with ≥ 1 test | 25 of 25 (via ISO 13849-2 matrix) |
| DFMEA modes with ≥ 1 test | 29 of 32 directly + 3 indirectly |
| Reqs with ≥ 1 test | 120 of 120 (100 %) |
| Standards-clauses 'met' (concept) | 5 of 30 (16.7 %); 7 partial; 16 not met; 2 deferred |
| Open gaps | 28 (10 Very-High, 9 High, 7 Med, 2 Low) |

The concept-phase package is internally consistent. Closing the 11 Very-High DFMEA actions and the 10 Very-High safety-case gaps is the precondition for OE-1 unconditional acceptance and OE-2 conditional acceptance. OE-3 / OE-4 require additional regulatory engagement beyond the scope of this revision.

6. Artefact index

| File | Type | Purpose |
| --- | --- | --- |
| Asimov_v1_Safety_Case.xlsx | Workbook | 15 tabs — full safety case |
| Asimov_v1_Safety_Case_Report.docx | Doc | Narrative safety case (9 sections) |
| Asimov_v1_Safety_Case_Summary.pdf | PDF | 2-page executive summary |
| Asimov_v1_DFMEA.xlsx | Workbook | AIAG-VDA 2019 DFMEA (12 tabs, 32 modes) |
| Asimov_v1_Requirements.xlsx | Workbook | SyRS+SRS+HwRS (9 tabs, 120 reqs) |
| Asimov_v1_Architecture.docx | Doc | Five architectural views with embedded diagrams |
| Asimov_v1_Diagrams/01..05 .svg + .png | Images | Architecture diagrams (vector + raster) |
| Asimov_v1_TestPlan.xlsx | Workbook | V-model test plan (10 tabs, 139 cases) |
| Asimov_v1_Engineering_Bundle.pdf | PDF | Stakeholder bundle (arch + reqs + DFMEA + tests + gaps) |
| Asimov_v1_Trace_Memo.docx | Doc | This memo |