ASIMOV v1

PRELIMINARY SAFETY CASE — CONCEPT PHASE

Open-Source Humanoid Robot · 1.2 m · 35 kg · 25 actuated DOF

Standards Framework Applied

| Reference | Coverage |
| --- | --- |
| ISO 12100:2010 | Safety of machinery — risk assessment and risk reduction (master) |
| ISO 10218-1/-2:2011 | Industrial robot safety (Part 1 robot, Part 2 system & integration) |
| ISO/TS 15066:2016 | Collaborative robot — power-and-force-limiting (PFL) data |
| ISO 13849-1:2023 | Safety-related parts of control systems — PL determination |
| ISO 13849-2:2012 | Validation |
| ISO 13482:2014 | Personal-care robot — Type 1 mobile servant hazard catalogue |
| ISO 9001:2015 | Quality management — design control, NCR, CAR |
| IEC 60204-1:2018 | Electrical equipment of machines (referenced) |
| IEC 62133-2:2017 | Li-ion battery safety (referenced) |

Document Control

| Field | Value |
| --- | --- |
| Document ID | ASV1-SC-CP-001 |
| Revision | 0.1 (draft) |
| Issue date | 2026-05-03 |
| Author | J. Herrod-Thomas (forker / assessor) |
| Item under assessment | Asimov v1 — fork at github.com/jherrodthomas/asimov-v1 |
| Hardware licence | CERN-OHL-S-2.0 |
| Software licence | GPL-2.0 |
| Confidentiality | Public — distributed under reciprocity |

Important — preliminary safety case This document does NOT certify Asimov v1 as safe to deploy. The upstream release publishes mechanical CAD, electrical schematics, and a MuJoCo model only — no HARA, FMEA, FMEDA, controller code, E-Stop validation, or battery management specification is published. The argument structure is a SCAFFOLD populated with concept-level evidence; every leaf claim links to the Gap Register in the companion workbook. Deployment beyond a supervised, tethered laboratory setting requires closure of high-severity gaps before any safety conclusion can be drawn.

1. Executive Summary

Asimov v1 is an open-source bipedal humanoid robot developed by Asimov Inc. (Menlo). This safety case applies the ISO 12100 master risk-assessment method, augmented by the control-system performance-level (PL) framework of ISO 13849-1, the personal-care hazard catalogue of ISO 13482, the industrial-robot envelope of ISO 10218 and ISO/TS 15066, and the quality-management framework of ISO 9001.

Thirty-two hazards have been identified across mechanical, electrical, thermal, environmental, personal-care, and cyber/control categories. Twenty-five safety functions have been derived to mitigate them. Of the 30 standards-clauses examined: 5 are met at concept level, 7 are partial, 16 are not met, and 2 are deferred to the deploying organisation.

Headline finding

Asimov v1 is provisionally acceptable for OE-1 (research lab, supervised, tethered) only. Deployment to industrial cobot environments requires closure of 12 gaps; deployment to home or public-space environments — the operating modes that ISO 13482 was written for — requires closure of 19 and 21 gaps respectively. The most critical gaps are: wireless E-Stop architecture and validation, joint force/energy limit declaration, MTTFD/DC/CCF figures for control-system PL computation, locomotion-policy verification, and battery (BMS) compliance gating in the DIY kit.

Risk profile by operational envelope

| Op envelope | Description | Risk | Provisional acceptance |
| --- | --- | --- | --- |
| OE-1 Lab (supervised) | Trained operator present, tethered/fixtured, no bystanders | LOW | PROVISIONAL YES |
| OE-2 Industrial cobot | Trained users, defined cell, possible collaboration | MEDIUM | NOT YET — 12 gaps |
| OE-3 Home / consumer | Untrained users, mixed surfaces, children/pets | HIGH | NOT YET — 19 gaps |
| OE-4 Public space | Bystanders, weather, kerbs, elderly/disabled | VERY HIGH | NOT YET — 21 gaps |

2. Scope and Item Definition

2.1 Item under assessment

Asimov v1 is a 1.2 m, 35 kg bipedal humanoid robot with 25 actuated degrees of freedom plus 2 passive (legs 6 DOF × 2 + toe × 2; arms 5 DOF × 2; torso 1 DOF waist yaw; neck 2 DOF). The structure is 7075 aluminium and HP Multi-Jet-Fusion PA12 nylon. On-board compute is split: a Raspberry Pi 5 handles media and network, a Radxa CM5 (mounted on the Motion Control Board) handles motion control. Six CAN buses partition actuator traffic (left leg, right leg, left arm, right arm, waist-neck, BMS). Sensing is a 6-DOF LSM6DSV IMU, a 2 MP monocular camera, a quad microphone array, and joint-state telemetry.

Specified mechanical capability is squat 5 kg, bicep curl 15 kg per arm, lateral raise 18 kg per arm. The DIY kit excludes the battery — a critical safety boundary discussed in section 8.

2.2 Functions of the item

| ID | Function |
| --- | --- |
| F-01 | Stand — maintain bipedal balance |
| F-02 | Walk — forward / backward / turn locomotion |
| F-03 | Manipulate — upper-body reach (no end-effector in v1) |
| F-04 | Sense — stream camera, audio, IMU |
| F-05 | Tele-op — accept commands over WebRTC |
| F-06 | Damp — passive limp state on disconnect / watchdog timeout |
| F-07 | Stand-up-from-pose — ramp to standing |
| F-08 | E-Stop — remove motor power on wireless trigger |

2.3 Operating modes (declared)

| Mode | Behaviour | Entry condition |
| --- | --- | --- |
| M-01 DAMP | Motors limp; safe default | On disconnect / watchdog timeout / E-Stop |
| M-02 STAND | Controller maintains standing pose; ramps over ~2 s | ModeCommand(STAND) |
| M-03 MOVE | Accepts position/torque commands | ModeCommand(MOVE) after STAND established |

2.4 Use limits and reasonably foreseeable misuse

Per ISO 12100 §5.3, reasonably foreseeable misuse considered in this case includes: operation by untrained users, operation around children or pets, operation outdoors on uneven terrain, operation without the wireless E-Stop in the operator's reach, operation with damaged 7075 structural parts, charging or powering with non-OEM battery packs, and modification of joint torque limits in software.

2.5 Assumptions explicitly relied upon

| ID | Assumption |
| --- | --- |
| A-01 | Operator is trained and within E-Stop range at all times in MOVE. |
| A-02 | Battery (when fitted) is UN 38.3 / IEC 62133-2 compliant Li-ion with integrated BMS. |
| A-03 | Structure is genuine 7075 Al and MJF PA12; no substitutions during DIY build. |
| A-04 | CAN harness is built per WireViz with correct termination and crimp. |
| A-05 | Operating surface is level (≤5°), dry and free of obstacles. |
| A-06 | No human body part enters the swept volume of any actuated joint while in M-02 or M-03. |
| A-07 | Robot is NOT a medical device, child-care robot, or load-bearing person-carrier. |

3. Standards Compliance Summary

Thirty clauses across six standards (and three referenced) were examined. The complete matrix lives in tab 04_StdMatrix of the companion workbook; the headline distribution is below.

| Standard | Clauses examined | Met (concept) | Partial | Not met | Deferred |
| --- | --- | --- | --- | --- | --- |
| ISO 12100 | 5 | 2 | 2 | 1 | 0 |
| ISO 10218-1/-2 + TS 15066 | 5 | 0 | 0 | 4 | 1 |
| ISO 13849-1/-2 | 4 | 1 | 1 | 2 | 0 |
| ISO 13482 | 7 | 1 | 1 | 5 | 0 |
| ISO 9001 | 6 | 0 | 1 | 5 | 0 |
| IEC 60204-1 | 2 | 0 | 2 | 0 | 0 |
| IEC 62133-2 | 1 | 0 | 0 | 0 | 1 |
| TOTAL | 30 | 5 | 7 | 16 | 2 |

The cluster of 'Not met' findings sits primarily on the V&V evidence side: there is no published test report, no MTTFD or DC figure for any control channel, and the wireless E-Stop architecture is not specified. The ISO 9001 cluster reflects that the upstream project uses git for revision control but does not maintain design-review records, NCR or CAR systems publicly.

4. Hazard Analysis Findings

4.1 Catalogue construction

The catalogue was built by traversing ISO 12100 Annex B (general machinery hazards) and ISO 13482 Annex A (personal-care robot hazards) for completeness, then projecting each entry onto the Asimov v1 architecture. Thirty-two hazards survived as relevant; one (HZ-M06 high-pressure fluid injection) was excluded because the item has no fluid power.

4.2 Risk scoring method

Each hazard scenario is scored on the ISO 13849-1 Annex A risk graph: Severity (S1 reversible / S2 irreversible) × Frequency or exposure (F1 seldom / F2 frequent) × Possibility of avoidance (P1 possible / P2 scarcely possible). The graph yields a Required Performance Level (PLr) from a (lowest) to e (highest), which determines the architectural Category required of the safety control system.

4.3 Top hazards by residual risk

After applying the 25 declared safety functions (the FSC scaffold), 5 residual risks remain at High or Very-High and require closure before any deployment beyond OE-1.

| Hazard | Scenario | PLr | Residual | Reason |
| --- | --- | --- | --- | --- |
| HZ-T02 | Li-ion thermal runaway (battery) | d | VERY HIGH | BMS user-supplied; not gated by kit |
| HZ-P02 | Locomotion policy edge case → hazardous traj. | e | HIGH | Policy unreleased; sim2real gap |
| HZ-P04 | Intentional contact exceeds force limit | e | HIGH | PFL thresholds not declared (TS 15066) |
| HZ-M02 | Joint pinch during MOVE | e | MED | Speed limits not declared; perception not in v1 |
| HZ-M04 | Arm sweep strike | e | MED | Speed reduction trigger not implemented |

4.4 Hazards by category

| Category | Count | Notes |
| --- | --- | --- |
| Mechanical | 8 | Tip-over, pinch, shear, sharp-edge, entanglement, stability, slip |
| Electrical | 4 | Shock, arc, ESD, EMI |
| Thermal | 2 | Stator burn, battery runaway |
| Noise | 1 | Speaker > 85 dBA at user ear |
| Materials/dust | 1 | MJF PA12 inhalation during repair |
| Ergonomic | 1 | 35 kg manual lift |
| Environmental | 2 | Out-of-temp range, wet/wet floor |
| Personal-care (13482) | 7 | Restraint, autonomous decision, power loss, contact, lean, cognitive |
| Control / cyber | 5 | Comms loss, unauth cmd, IMU stuck, deadlock, joint overshoot |

5. Safety Argument (Goal Structuring Notation, narrative)

The argument is structured top-down per the GSN community standard. The top-level Goal G1 claims that Asimov v1 is acceptably safe within the declared envelope. It is decomposed by Strategy S1 into four sub-arguments — hazard completeness, three-step risk reduction, safety-function integrity, and standards conformance — each leading to leaf Solutions or open Goals.

5.1 Top-level claim

G1 — Top Goal Asimov v1 is acceptably safe within the declared operational envelope (OE-1 lab supervised) and all reasonably foreseeable misuse is mitigated.

5.2 Decomposition

Strategy S1 — argue over hazard identification completeness, three-step risk reduction (ISO 12100 §6.5), safety-function integrity (ISO 13849-1), and standards conformance — produces four sub-goals (G2–G5) plus a quality-management leg (G6) and a personal-care-specific leg (G7).

G2 — All foreseeable hazards identified

Discharged at concept level by Sn1: ISO 12100 Annex B + ISO 13482 Annex A traversed; 32 hazards catalogued (tab 06).

G3 — Risk reduced to acceptable

Decomposed by Strategy S3 into G3.1 inherent design (mass/edges/joint travel — partially evident in upstream CAD), G3.2 safeguarding via 25 safety functions (FSC scaffold, tab 09), and G3.3 information-for-use (NOT evident — manual subpages /v1/safety, /v1/quickstart return 404, see GAP-DOC-01).

G4 — Each safety function meets its required PL

Decomposed per ISO 13849-1 §4.5.4 into G4.1 Category (target allocated, tab 09), G4.2 MTTFD per channel (NOT evident — GAP-13849-02), G4.3 DC per channel (NOT evident — GAP-13849-02), G4.4 CCF ≥ 65 (NOT evident — GAP-13849-02), G4.5 Validation per ISO 13849-2 (NOT evident — GAP-13849-03). G4 therefore CANNOT be discharged at this revision.

G5 — Standards conformance demonstrated

Sn5 produces the standards-compliance matrix (tab 04). At present 6 of 33 clauses are met at concept level; G5 is partially supported but cannot be discharged.

G6 — ISO 9001 quality management

Git-backed document control is acceptable. Design-review records, NCR procedure, CAR system, internal audit and monitoring are absent (GAP-9001-01 through 06). G6 NOT discharged.

G7 — Personal-care robot specifics

Six gaps remain in ISO 13482 §5.7 (force/energy limits), §5.10 (speed/torque/momentum), §5.13 (stability), §5.16 (E-Stop), §6 (control system), §7 (V&V). Deployment to OE-3/OE-4 NOT recommended at this revision.

5.3 Conclusion node

CONCL — Conclusion Provisional acceptance for OE-1 (lab supervised) only, contingent on closure of: GAP-10218-03 (E-Stop architecture), GAP-13849-02 (PL evidence), GAP-13482-01 (force/energy), GAP-13849-03 (validation), GAP-BMS-01 (battery gate). Deployment to OE-2/3/4 BLOCKED.

6. Functional Safety Concept — Safety Functions

Twenty-five safety functions are declared. Each is allocated to a hardware/software/procedural owner and assigned a Required Performance Level per ISO 13849-1. The full FSC sits in tab 09 of the workbook; selected high-PL functions are summarised below.

| SF ID | Function | PLr | Cat | Reaction | Allocation |
| --- | --- | --- | --- | --- | --- |
| SF-01 | Maintain bipedal stability | d | 3 | ≤ 20 ms | Loco. policy + IMU + MCB |
| SF-02 | Detect & arrest impending fall (controlled descent) | d | 3 | ≤ 200 ms | MCB + IMU + actuators |
| SF-03 | Joint torque/velocity limit per joint (per TS 15066) | d | 3 | ≤ 10 ms | MCB FW + drive limits |
| SF-04 | Force-limited contact in collaborative regions | d | 3 | ≤ 100 ms | MCB FW + sensorised links |
| SF-08 | Safe-stop on watchdog timeout (Cat 1 stop) | d | 3 | ≤ 100 ms | MCB watchdog |
| SF-13 | Wireless E-Stop link surveillance + heartbeat | d | 3 | ≤ 500 ms | E-Stop receiver |
| SF-15 | Battery thermal & cell-voltage protection (BMS) | e | 4 | ≤ 1 s | BMS (user-supplied) |
| SF-20 | Power-and-Force Limiting per ISO/TS 15066 | d | 3 | ≤ 100 ms | MCB FW + design |
| SF-23 | IMU plausibility & cross-check | d | 3 | ≤ 50 ms | MCB FW |
| SF-24 | Real-time scheduler watchdog & deadline monitor | d | 3 | ≤ 50 ms | MCB FW |

Of these, SF-15 (BMS) requires PL e / Cat 4 — currently delegated to a user-supplied battery pack with no compliance gate, the single largest residual risk. SF-13 (wireless E-Stop) is declared but the underlying architecture and link-surveillance scheme is not published.

7. Residual Risk and Deployment Guidance

Residual risk after concept-level safety functions remains High or Very-High for five hazards (see §4.3). Deployment guidance is therefore conditional on the operational envelope:

7.1 OE-1 Lab (supervised)

Provisionally permitted Tethered or fixtured operation, trained operator within wireless E-Stop range, no bystanders, indoor +20–25 °C, level dry surface, demonstration speed only. Builder must verify all 7 assumptions in §2.5 and have a working DAMP fallback.

7.2 OE-2 Industrial cobot

Not yet permitted — 12 gaps to close Requires closure of GAP-10218-01/02/03 (single-fault tolerance, PLd Cat 3 validation, E-Stop architecture), GAP-15066-01 (PFL thresholds), GAP-13849-02/03 (PL evidence and validation), GAP-13482-01/02 (force/energy and speed declaration), GAP-DOC-01 (assembly/safety manual), GAP-BMS-01 (battery gate), GAP-LP-01 (locomotion policy), GAP-API-01 (API stability).

7.3 OE-3 Home / consumer

Not yet permitted — 19 gaps to close Adds all of OE-2 plus ISO 13482 §5.13 stability under person-leaning, §5.7 contact energy with untrained users including children/pets, environmental robustness (no IP rating), cyber authentication for tele-op, and full ISO 9001 quality system. The locomotion policy must be verified for the long tail of household scenarios.

7.4 OE-4 Public space

Not permitted at any near-term revision — 21 gaps Free-roaming around the public is the highest-hazard envelope ISO 13482 covers. In addition to all OE-3 gaps, the project would need: weather/IP rating, GNSS/odometry fault tolerance, dynamic obstacle avoidance with verified worst-case latency, regulatory engagement (CE / FCC / national robotics frameworks), liability insurance, and an incident-reporting system. Recommend explicit prohibition in the v1 manual.

8. Gap Register — top 12 gaps to close

Twenty-eight gaps are tracked in tab 12 of the workbook. The top 12 by severity are summarised here; closing these unlocks OE-1 deployment unconditionally and OE-2 conditionally.

| ID | Standard / clause | Description | Severity |
| --- | --- | --- | --- |
| GAP-13482-04 | ISO 13482 §5.16 | Wireless E-Stop architecture, latency, link surveillance | VERY HIGH |
| GAP-10218-03 | ISO 10218-1 §5.5.3 | E-Stop architecture (Cat 0/1, dual-channel) | VERY HIGH |
| GAP-13849-02 | ISO 13849-1 §7 | MTTFD/DC/CCF figures absent — PL cannot be computed | VERY HIGH |
| GAP-13849-03 | ISO 13849-2 | Validation plan & test report missing | VERY HIGH |
| GAP-13482-01 | ISO 13482 §5.7 | Force/energy limits not declared | VERY HIGH |
| GAP-13482-05 | ISO 13482 §6 | Safety-related control-system implementation gaps | VERY HIGH |
| GAP-13482-06 | ISO 13482 §7 | V&V plan and report not produced | VERY HIGH |
| GAP-13849-01 | ISO 13849-1 §6 | Cat 3 redundancy not implemented for 10 SFs | VERY HIGH |
| GAP-BMS-01 | IEC 62133-2 | Battery is user-supplied — no compliance gate | VERY HIGH |
| GAP-LP-01 | (this case) | Locomotion policy unreleased | VERY HIGH |
| GAP-12100-01 | ISO 12100 §6.4 | Adequacy-of-risk-reduction record absent for 18 hazards | HIGH |
| GAP-DOC-01 | (this case) | Manual /v1/safety, /v1/quickstart, /v1/bom return 404 | HIGH |

9. Conclusion

Asimov v1 is a serious, well-structured open-source robotics project. The mechanical CAD release, fab-ready KiCad sources, and MuJoCo simulation model establish a credible foundation. However, the safety-relevant evidence base — HARA, FMEA, FMEDA, control-system PL validation, E-Stop architecture, battery management, locomotion-policy verification, and quality-management records — is not yet present in the public release. This concept-phase safety case provides a scaffolded argument over what does exist and a register of what does not.

Provisional acceptance is granted for OE-1 (research-lab, supervised, tethered) only. Deployment to industrial, home, or public-space environments is not recommended pending closure of the gaps identified.

9.1 Recommended next-revision actions

1. Publish wireless E-Stop architecture (Cat 1 stop, dual-channel, link surveillance, fault-reaction time) per ISO 13849-1 / ISO 13482 §5.16.

2. Declare joint torque, velocity and momentum limits per joint, with verification report, per ISO 13482 §5.10.

3. Publish a power-and-force-limiting (PFL) measurement campaign per ISO/TS 15066 for any planned cobot use.

4. Compute and publish MTTFD, DC, and CCF for each safety-rated control channel; demonstrate PL achieved per ISO 13849-1 §7.

5. Issue an ISO 13849-2 validation plan and test report covering the 25 safety functions.

6. Gate the DIY kit against a list of pre-qualified Li-ion packs that meet UN 38.3 and IEC 62133-2 with a compliant BMS.

7. Restore manual subpages /v1/safety, /v1/quickstart, /v1/bom with explicit operating-envelope and warning content per ISO 12100 §6.5.3.

8. Stand up an ISO 9001-aligned QMS subset: design-review records, NCR/CAR procedure, supplier control, internal audit.

9. Verify the locomotion policy against the 32-hazard catalogue and publish a sim-to-real evidence pack including domain-randomisation coverage and worst-case-latency budgets.

10. Specify and authenticate the tele-op channel (TLS, token, integrity protection, idle-timeout) before exposing the Asimov API.

9.2 References

Standards: ISO 12100:2010, ISO 10218-1/-2:2011, ISO/TS 15066:2016, ISO 13849-1:2023, ISO 13849-2:2012, ISO 13482:2014, ISO 9001:2015, IEC 60204-1:2018, IEC 62133-2:2017, ISO 14971:2019 (reference). Project: github.com/jherrodthomas/asimov-v1 (fork of asimovinc/asimov-v1); manual.asimov.inc/v1. Companion workbook: Asimov_v1_Safety_Case.xlsx.